##### Date: 			1 May 2006

# Release for IP Softphone 2050 Version 2

IPSoftphone2050_Settings.exe

Revision History

| Date | Revision # | Summary of Changes |
| --- | --- | --- |
| 1-May-06 | Original | This is the original publication |

## Introduction

The IPSoftphone2050_Settings is a utility which can be run to configure a IP Softphone 2050.

To use this utility:

- Download the IP Softphone 2050 setup exe to the user’s PC

- Install the IP Softphone 2050, using the setup exe, on the user’s computer

- Edit the 2050Settings.ini file to define the configuration required for the end user

- Download the IPSoftphone2050_Settings.exe and the 2050Settings.ini file to the user’s computer

- Customize the 2050Settings.ini as required

- In the user’s runtime context, run the IPSoftphone2050_Settings.exe specifying the 2050Settings.ini file as the input file.

Example: “IPSoftphone2050_Settings.exe 2050Settings.ini”

IPSoftphone2050_Settings writes the settings defined in the input file to the registry where the IP Softphone 2050 will subsequently read them.

Each 2050Settings.ini file contains settings for one profile for a IP Softphone 2050 user. If the registry settings in the .ini file have not been previously assigned, the utility will create them. If the registry settings in the .ini file have been previously assigned, the utility will overwrite them with the new values. If the administrator wishes to create more than one profile for a user, then additional .ini files must be created and the IPSoftphone2050_Settings.exe run multiple times, one for each profile. The last .ini file used with a setting “Default=1” defines the profile which will be the default profile for the user.

The settings in the 2050Settings.ini file are assumed to be correct. Since this is an administration utility little error checking is performed.

The .ini file is divided into sections for collections of various attributes. These sections, in general, correspond to the various settings pages in IP Softphone 2050 Settings (Start > Programs > Nortel > IP Softphone 2050 > IP Softphone 2050 Settings). An example 2050settings.ini file is provided and is included below. The file format is subsequently described in detail below.

==== START OF 2050Settings.ini file example ====

[Profile]

Name=Lab M1

Default=1

ShowProfilesOnStartup=1

[UserPIN]

Node=4001

TN=63-5

[Server1]

; set address=0 to use DHCP

address=47.9.11.70

symposium=0

name=

port=4100

retries=10

; Meridian 1, BCM, CS 1000, CS 1000M

type=CS 1000

[Server2]

address=47.9.11.71

symposium=0

name=

port=7000

retries=11

; Meridian 1, BCM, CS 1000, CS 1000M

type=BCM

[ListenerIP]

IP=1.2.3.4

Port=1234

[Skin]

; 1140E, Silver, Black

Name=1140E

Accessibility=0

[Language]

; English, Czech, Danish, German, Spanish, Finnish, French, Hungarian

; Italian, Japanese, Latvian, Dutch, Norwegian, Polish, Portuguese

; Russian, Swedish, Turkish

Name=English

[KeyType]

Line1=1

Line2=0

Line3=0

Line4=0

Line5=0

Line6=0

[USBHeadset]

; GN Netcom DuraPlus Binaural Headset

; GN Netcom DuraPlus Monaural Headset

; GN Netcom GN2100 Monaural Headset

; GN Netcom GN2115 ST Binaural Headset

; GN Netcom GN2117 ST Monaural Headset

; GN Netcom GN2125 NC Binaural Headset

; GN Netcom GN2220 NC Monaural Headset

; GN Netcom GN2225 NC Binaural Headset

; Nortel Networks Handset

; Nortel Networks Mobility Kit

HeadsetType=GN Netcom DuraPlus Binaural Headset

; None, On, Flash, Flicker, Blink

AlertConditionIndication=

; None, On, Flash, Flicker, Blink

ActiveCallIndication=

; None, On, Flash, Flicker, Blink

MessageWaitingIndication=

; None, On, Flash, Flicker, Blink

HeadsetDisconnectedIndication=

UseBacklight=0

==== END OF 2050Settings.ini file example ====

Section: [Profile]

Attribute: Name

Settings Tab:	Profile

Description: 	The name of the profile. All other attributes in the file will be applied for this profile.

Format/Values:	String

Example: 	Name=Lab M1

Attribute: Default

Settings Tab:	Profile

Description: 	Is this profile the to be used as the default profile?

Format/Values:	1 - for True

0 - for False

Example: 	Default=1

Attribute: ShowProfilesOnStartup

Settings Tab:	None

Description: 	Allow the user to select a profile at startup?

Format/Values:	1 - for True

0 - for False

Example: 	ShowProfilesOnStartup=0

Sub-Section: [UserPIN]

Attribute: Node

Settings Tab:	None

Description: 	CS1000 Node id

Format/Values:	Numeric

Example: 	Node=4001

Attribute: TN

Settings Tab:	None

Description: 	CS1000 TN value

Format/Values:	Numeric - use the same format in which CS1000 displays/requests the TN

Example: 	TN=63-5

Section: [Server1]

Attribute: address

Settings Tab:	Server: Primary

Description: 	IP Address

Format/Values:	IP Address format

Example: 	address=47.9.11.70

Notes:	To use DHCP to provide an address, use address=0

Attribute: symposium

Settings Tab:	Server: Primary

Description: 	Is this client is connecting to a CS1000 server and is being used as a symposium client? Ie: should the client report the handset as being off-hook.

Format/Values:	1 - for True

0 - for False

Example: 	symposium=0

Attribute: name

Settings Tab:	Server: Primary

Description: 	DNS Name. This can be used instead of the IP Address

Format/Values:	DNS name string

Example: 	cs1000.us.nortel.com

Attribute: port=4100

Settings Tab:	Server: Primary

Description: 	The TPS connection port.

Format/Values:	Numeric

Example: 	port=4100

Attribute: retries=10

Settings Tab:	Server: Primary

Description: 	If a connection with a TPS fails, the client will retry this number of times before attempting to connect to [Server2]

Format/Values:	Numeric

Example: 	retries=10

Attribute: type=CS 1000

Settings Tab:	Server: Primary

Description: 	TPS server type

Values:	Meridian 1, BCM, CS 1000, CS 1000M

Example: 	type=CS 1000

Section: [Server2]

Server 2 is the alternate server. This is called S2 for the 1004/2004. If connection to Server1 (S1) fails, the client will attempt to connect to this server.

Attribute: address

Settings Tab:	Server: Secondary

Description: 	See [Server1] address

Attribute: symposium

Settings Tab:	Server: Secondary

Description: 	See [Server1] symposium

Attribute: name

Settings Tab:	Server: Secondary

Description: 	See [Server1] name

Attribute: port

Settings Tab:	Server: Secondary

Description: 	See [Server1] port

Attribute: retries

Settings Tab:	Server: Secondary

Description: 	See [Server1] retries

Attribute: type

Settings Tab:	Server: Secondary

Description: 	See [Server1] type

Section: [ListenerIP]

Listener IP can be used to override port assignments when the IP Softphone 2050 application conflicts with another application on your computer.

Under normal operation, the IP Softphone 2050 listens for UNIStim traffic on all IP addresses and IP ports. You can set the IP Softphone 2050 to listen to only one specific IP address and/or port.

Attribute: IP

Settings Tab:	ListenerIP

Description: 	The IP address where the IP Softphone 2050 will listen for UNIstim (signaling) traffic. This must be an IP address on the client’s computer.

Format/Values:	IP address format

Example: 	IP=47.25.89.123

Attribute: port

Settings Tab:	ListenerIP

Description: 	The port where the IP Softphone 2050 will listen for UNIstim (signaling) traffic.

Format/Values:	Numeric

Example: 	port=5000

Section: [Skin]

Attribute: Name

Settings Tab:	Skin

Description: 	The user interface selection

Format/Values:	1140E, Silver, Black

Example: 	Name=1140E

Attribute: Accessibility

Settings Tab:	Skin

Description: 	Should the accessibility interface be used? Setting this value to True will ignore the Skin Name above.

Format/Values:	1 - for True

0 - for False

Example: 	Accessibility=0

Section: [Language]

Attribute: Name

Settings Tab:	Language

Description: 	Select the interface language. Note: this does not affect the language used in the server display area.

Format/Values:	English, Czech, Danish, German, Spanish, Finnish, French, Hungarian, Italian, Japanese, Latvian, Dutch, Norwegian, Polish, Portuguese, Russian, Swedish, Turkish

Example: 	Name=English

Section: [KeyType]

These are the line keys which are used for calls (ie: not keys which are configured as feature keys).

Attribute: Line1

Settings Tab:	KeyType

Description: 	Is this line used for calls?

Format/Values:	1 - for True

0 - for False

Example: 	Line1=1

Attribute: Line2=0

Settings Tab:	KeyType

Description: 	Is this line used for calls?

Format/Values:	1 - for True

0 - for False

Example: 	Line2=0

Attribute: Line3=0

Settings Tab:	KeyType

Description: 	Is this line used for calls?

Format/Values:	1 - for True

0 - for False

Example: 	Line3=0

Attribute: Line4=0

Settings Tab:	KeyType

Description: 	Is this line used for calls?

Format/Values:	1 - for True

0 - for False

Example: 	Line4=0

Attribute: Line5=0

Settings Tab:	KeyType

Description: 	Is this line used for calls?

Format/Values:	1 - for True

0 - for False

Example: 	Line5=0

Attribute: Line6=0

Settings Tab:	KeyType

Description: 	Is this line used for calls?

Format/Values:	1 - for True

0 - for False

Example: 	Line6=0

Section: [USBHeadset]

Attribute: HeadsetType

Settings Tab:	USBHeadset

Description: 	Which headset is being used?

Format/Values:	GN Netcom DuraPlus Binaural Headset

GN Netcom DuraPlus Monaural Headset

GN Netcom GN2100 Monaural Headset

GN Netcom GN2115 ST Binaural Headset

GN Netcom GN2117 ST Monaural Headset

GN Netcom GN2125 NC Binaural Headset

GN Netcom GN2220 NC Monaural Headset

GN Netcom GN2225 NC Binaural Headset

Nortel Networks Handset

Nortel Networks Mobility Kit

Example: 	GN Netcom DuraPlus Binaural Headset

Attribute: AlertConditionIndication

Settings Tab:	USBHeadset

Description: 	When an external lamp is in use, this value defines the flash frequency of that lamp - when a call is alerting. Note that this functionality requires an optional Nortel certified third party lamp module.

Format/Values:	None, On, Flash, Flicker, Blink

Example: 	AlertConditionIndication=Flash

Attribute: ActiveCallIndication

Settings Tab:	USBHeadset

Description: 	When an external lamp is in use, this value defines the flash frequency of that lamp - when a call (speech path) is active. Note that this functionality requires an optional Nortel certified third party lamp module.

Format/Values:	None, On, Flash, Flicker, Blink

Example: 	ActiveCallIndication=On

Attribute: MessageWaitingIndication

Settings Tab:	USBHeadset

Description: 	When an external lamp is in use, this value defines the flash frequency of that lamp - when the message waiting lamp is on. Note that this functionality requires an optional Nortel certified third party lamp module.

Format/Values:	None, On, Flash, Flicker, Blink

Example: 	MessageWaitingIndication=Flash

Attribute: HeadsetDisconnectedIndication

Settings Tab:	USBHeadset

Description: 	When an external lamp is in use, this value defines the flash frequency of that lamp - when the headset is disconnected from the USB adapter. Note that this functionality requires an optional Nortel certified third party lamp module.

Format/Values:	None, On, Flash, Flicker, Blink

Example: 	HeadsetDisconnectedIndication=Blink

Attribute: UseBacklight=0

Settings Tab:	USBHeadset

Description: 	Turn the USB adapter’s lamp on when the IP Softphone 2050 is connected to a server.

Format/Values:	1 - for True

0 - for False

Example: 	UseBacklight=0

## IP Phone Compatibility

This utility (IPSoftphone2050_Settings.exe) is applicable to IP Softphone 2050 Version 2.x (also known as IP Softphone 2050 Version 2).

## Call Server Compatibility and Requirements

The Node and TN attributes are applicable only to CS1000, CS1000M and Meridian 1 servers.

All other attributes are applicable to all server types and versions which support IP Softphone 2050 Version 2.

Nortel is a recognized leader in delivering communications capabilities that enhance the human experience, ignite and power global commerce, and secure and protect the world’s most critical information. Serving both service provider and enterprise customers, Nortel delivers innovative technology solutions encompassing end-to-end broadband, Voice over IP, multimedia services and applications, and wireless broadband designed to help people solve the world’s greatest challenges. Nortel does business in more than 150 countries. For more information, visit Nortel on the Web at www.nortel.com.